Samson Mano

saminnx@gmail.com

https://sites.google.com/site/samsoninfinite/

https://github.com/Samson-Mano

EM WAVE PROPAGATION

Modelling oscillation of charged particle

# Introduction

## Charged particle

A charged particle is a particle with an electric charge. Electric charge (q) is the physical property of mater that causes it to experience a force when placed in an electromagnetic field. Electric charge can be positive or negative (commonly carried by protons and electrons respectively, by convention). Like charges repel each other and unlike charges attract each other. An object with no net charge is referred to as electrically neutral.

Coulomb's law can be stated as a simple mathematical expression. The scalar form gives the magnitude of the vector of the electrostatic force F between two point charges q1 and q2, but not its direction. If r is the distance between the charges, the magnitude of the force is

|  | Columb’s law: F=q1q24πε0r2 | 1.0 |
| --- | --- | --- |

where ε0 is the electric constant. If the product q1q2 is positive, the force between the two charges is repulsive; if the product is negative, the force between them is attractive.

## Electric field

An electric field is a vector field that associates to each point in space the Coulomb force experienced by a unit test charge. In the simplest case, the field is considered to be generated solely by a single source point charge.

|  | Electric field due to stationary point charge is: Er=q4πε0rr2 Where r is the radial unit vector. | 2.0 |
| --- | --- | --- |

## Electric potential

The electric potential, or voltage, is the difference in potential energy per unit charge between two locations in an electric field.

A charge q moving in a constant electric field E experience a force F = qE from that field. Gauss law states that the word done by the force is path independent.

|  | The difference in potential energy per unit charge between two locations in an electric field is given by, VB-VA=AB(∇V)∙dl= -ABE∙dl |  |
| --- | --- | --- |
|  | E= -∇V | 3.0 |
|  | The electric potential at point r is given by, Vr= -∞rE∙dl |  |
|  | = -∞rErdr |  |
|  | Vr=14πε0qr Where r is the distance from the charge. The path independency is due to the fact in electrostatics ∇×E=0. | 3.1 |
|  | The potential of collection of charges is given by, invoking superposition principle. Vr=14πε0i=1nqiri |  |
|  | Or for a continuous distribution Vr=14πε01rdq |  |
|  | For a volume charge the potential is given by Vr=14πε0ρr'rdτ Where ρ is the charge density | 3.2 |

## Electrostatics summary

## Magnetic field – Biot-Savart law

Biot-savart law is the describes the relationship between magnetic field and the electric current generating it. The Biot-Savart law allows calculating the value of magnetostatic field at any point in space, due to electrical current.

|  | Magnetic field due to electric current is: Br=μ04πI×rr2dl=μ0I4πdl×rr2 Where r is the radial unit vector. | 4.0 |
| --- | --- | --- |

The magnetic field of an infinite straight wire is the integral of B around a circular path of radius s, centered at the wire is,

|  | B∙dl=μ0I2πsdl=μ0I2πsdl=μ0I |  |
| --- | --- | --- |

For a bundle of straight wires shown below, the line integral will be

|  | B∙dl=μ0Ienc=μ0J∙da |  |
| --- | --- | --- |

Where Ienc is the total current enclosed by the integration path. If the flow of charge is represented by a volume current density J

|  | Applying stokes theorem. ∇×B∙da=μ0J∙da |  |
| --- | --- | --- |
|  | ∇×B=μ0J | 4.1 |
|  | The biot-savart law for general case of a volume current is given by, Br=μ04πJr'×rr2dτ | 4.2 |

## Magnetic vector potential

|  | Since ∇∙B=0 the magnetic vector potential can be described as B=∇×A | 5.1 |
| --- | --- | --- |
|  | Applying to the curl of B ∇×B=∇×∇×A=∇∇∙A-∇2A=μ0J |  |
|  | Given ∇∙A=0 ∇2A=-μ0J | 5.2 |
|  | The magnetic vector potential is given by, Ar=μ04πJr'rdτ | 5.3 |

## Magnetostatics summary

## Accelerating charged particle

Accelerating charges produce changing electric and magnetic fields.  Changing electric fields produce magnetic fields and changing magnetic fields produce electric fields.  This interplay between induced electric and magnetic fields leads to propagating electromagnetic waves.  Electromagnetic waves can propagate through free space.

Assume a charge q located near the origin is accelerating.  It therefore produces electromagnetic radiation.  At some position r in space and at some time t, the electric field of the electromagnetic wave produced by the accelerating charge is given by

|  | Electric field due to accelerating point charge is: Er,t=q4πε01rc2a⊥t-rc Where er is the radial unit vector. | 6.0 |
| --- | --- | --- |

## Derivation of the Larmor’s formula

Consider a charged particle which is at rest until time t = 0, experiences an acceleration a for an infinitesimal time interval ∆t, and then continues to move with uniform velocity.

In Region I the electric field lines have not yet received the information about the acceleration, so they will look as if the particle is at rest, and nothing has changed. However, the field lines in Region II have received the information that the particle has accelerated, so they will match up to the final position of the particle (point A).

|  | Solving for the acceleration yields vf=vi+a∆t v=0+a∆t a=v∆t | 7.0 |
| --- | --- | --- |

|  | The kink in the electric field lines in the electromagnetic waves that propagates through space. E⊥E∥=vtsinθ c∆t | 7.1 |
| --- | --- | --- |
|  | E⊥=vtsinθ c∆tq4πε01r2 |  |
|  | =vtsinθ c∆tq4πε01ctr |  |
|  | =v∆ttsinθ cq4πε01ctr |  |
|  | E⊥=q4πε01rc2asinθ | 7.2 |
|  | E⊥=q4πε01rc2a⊥t-rc | 7.3 |
|  | Where, a⊥t-cr=at-rcsinθ | 7.4 |

## Maxwell’s Equation

- Gauss law for Electric field

The Gauss theorem connects the ‘flow’ of electric field lines (flux) to the charges within the enclosed surface in simple terms. The net charge in the volume contained by a closed surface is exactly proportional to the net flux through the closed surface.

|  | ∇∙E=ρε0 ρ-Charge density, ε0-Permitivity of free space | 8.0 |
| --- | --- | --- |

The net electric flow stays 0 if no charges are contained by a surface. The number of electric field lines entering the surface equals the number of field lines exiting the surface.

- Gauss law for magnetic field

The net magnetic flux through any closed surface is equal to zero. There are no magnetic charges. Magnetic field lines always close in themselves. No matter how the (closed) Gaussian surface is chosen, the net magnetic flux through it always vanishes.

|  | ∇∙B=0 | 8.1 |
| --- | --- | --- |

- Differential form of Faraday’s law

A circulating electric field is produced by a magnetic field that changes with time.

|  | ∇×E=-∂B∂t | 8.2 |
| --- | --- | --- |

- Ampere – Maxwell’s law

An electric current or a changing electric flux through a surface produces a circulating magnetic field around any path that bounds that surface.

|  | A circulating magnetic field is produced by an electric current and by an electric field that changes with time. ∇×B=μ0J+ε0∂E∂t μ0-Magnetic permeability of free space J-Eletric current density in amperes per square meter Electric current density is proportional to the electric field. | 8.3 |
| --- | --- | --- |

## Delayed Potential

|  | In the static case, the Poisson’s equation of Electric scalar potential and magnetic vector potential, ∇2V= -1ε0ρ | 9.0 |
| --- | --- | --- |
|  | ∇2A= -μ0J |  |
|  | With the familiar solution Vr=14πε0ρr'rdτ | 9.1 |
|  | Ar=μ04πJr'rdτ | 9.2 |

The delayed time tr is the time when the electromagnetic field began to propagate from the point q(tr) where it is emitted to an observer at P. The objective is to calculate the delayed time tr from the current time t. The present location of the particle is q(t), when the signal is reached to P the observer will notice the signal is coming from the particle at q(tr).

|  | So, between the time t and tr (∆t=t-tr) the wave traveled the distance between q(tr) and P at the speed c. r-wtr=c∆t=c(t-tr) |  |
| --- | --- | --- |
|  | The delayed time is given by, tr=t-r-wtrc | 10.0 |

## Lienard – Wiechert Potential

|  | The delayed time for a charge source shown above is given by, tr=t-rc | 11.0 |
| --- | --- | --- |
|  | The non-static scalar and magnetic vector potentials are given by, Vr,t=14πε0ρr',trrdτ' | 11.1 |
|  | The current density of a rigid object is ρv Ar,t=μ04πJr',trrdτ'=μ04πρr',trvtrrdτ' |  |
|  | For a point charge q moving at a velocity v, the integral can be written as the sum of charge densities and ri is the distance to individual element. ρr',trrdτ≈iρi∆Viri |  |
|  | Since the ri≫a where a is the size of the charge density. The distance to the center of the charge can be taken as r so it can be taken out of the integral. So, we only need to find the integral of the following which is not equal to q. ρr',trdτ'≠q |  |
|  | The reason the integral of charge densities is not equal to the total charge is due to the distortion effect of moving charge. The potential from the charge is distorted due to its motion. The moving charge potential is higher if its moving towards the observer and lower if its moving away from the observer. |  |

|  | The figures above are an attempt to explain the geometrical distortion effects due to the velocity of particle and the information delay in reaching the observer. The interval it takes from the length information to travel to the observer the volume itself moves a distance. a'cosθc=a'-av Note that the length distortion is nothing to do with special relativity or Lorentz contraction. The length of the charge does not charge. The potential increase due to charge motion is reminiscent of Doppler effect. | 11.2 |
| --- | --- | --- |
|  | a'=a1-vccos θ | 11.3 |
|  | The distortion is only in the direction of the velocity, so the integral volume is. dτ'=dτ1-rvc |  |
|  | The dynamic scalar potential is given by, Vr,t=14πε0qcrc-r∙v | 11.4 |
|  | The magnetic vector potential is given by, Ar,t=μ04πvrρr',trdτ'=μ0v4πqcrc-r∙v | 11.5 |
|  | Vacuum permittivity, permittivity of free space, the electric constant ε0 is related to magnetic constant, vacuum permeability, permeability of free space by the following equation, ε0=1μ0c2 | 11.6 |
|  | Ar,t=vc2Vr,t | 11.7 |
|  | The above equations are the famous Lienard-Wiechert potentials for a moving point charge. |  |

## Fields of a moving point charge

|  | Differential form of Faraday’s law Eqn 8.2 ∇×E=-∂B∂t |  |
| --- | --- | --- |
|  | Applying the magnetic vector potential to the equation. ∇×E=-∂∂t∇×A | 12.0 |
|  | ∇×E+∂∂t∇×A=0 |  |
|  | ∇×E+∂A∂t=0 | 12.1 |
|  | Here is a quantity whose curl vanishes; therefore, it can be written as the gradient of a scalar. E+∂A∂t=-∇V | 12.2 |
|  | Substituting the above equation in the Gauss law Eqn 8.0 ∇∙E=ρε0 |  |
|  | ∇∙-∇V-∂A∂t=ρε0 |  |
|  | ∇2V+∂∂t∇∙A=-ρε0 | 12.3 |
|  | Substituting the Ampere – Maxwells equation Eqn 8.3 ∇×B=μ0J+ε0∂E∂t |  |
|  | ∇×∇×A=μ0J+ε0∂∂t-∇V-∂A∂t | 12.4 |
|  | ∇×∇×A=μ0J-μ0ε0∇∂V∂t-μ0ε0∂2A∂t2 |  |
|  | Using the below vector identity ∇×∇×A=∇∇∙A-∇2A | 12.5 |
|  | ∇∇∙A-∇2A=μ0J-μ0ε0∇∂V∂t-μ0ε0∂2A∂t2 |  |
|  | ∇2A-μ0ε0∂2A∂t2-∇∇∙A+μ0ε0∂V∂t=-μ0J | 12.6 |
|  | Equation 12.3 and Equation 12.6 contains all the information of Maxwell’s Equation. |  |

Electric and magnetic fields of a point charge in arbitrary motion, using Lienard-Wiechert potentials.

|  | Re-writing the Lienard-Wiechert potential Eqn 11.4 & 11.7 Vr,t=14πε0qcrc-r∙v Ar,t=vc2Vr,t |  |
| --- | --- | --- |
|  | Re-writing the electric field equation Eqn 12.2 E=-∇V-∂A∂t | 13.0 |
|  | Magnetic field equation Eqn 5.1 B=∇×A | 13.1 |

|  | The vector from the delayed position to the observation position and the velocity of the charge is given by, r=r-wtr v=wtr | 13.2 |
| --- | --- | --- |
|  | Delayed time equation Eqn 10.0 r=r-wtr=c(t-tr) | 13.3 |
|  | To solve the Eqn 13.0, Let start by finding the gradient of V ∇V=∇14πε0qcrc-r∙v | 13.4 |
|  | ∇V=qc4πε0-1rc-r∙v2∇rc-r∙v | 13.5 |
|  | Finding below gradient ∇rc-r∙v=c∇r-∇r∙v | 13.6 |
|  | First term c∇r=c∇r-wtr |  |
|  | =-c∇wtr | 13.7 |
|  | =-c∂wx∂x+∂wy∂y+∂wz∂z |  |
|  | Second term (applying product rule) ∇r∙v=r∙∇v+v∙∇r+r×∇×v+v×∇×r | 13.8 |
|  | Term 1 of Eqn 13.8 r∙∇v=rx∂∂x+ry∂∂y+rz∂∂zvtr |  |
|  | =rx∂vtr∂x+ry∂vtr∂y+rz∂vtr∂z |  |
|  | =rx∂v∂tr∂tr∂x+ry∂v∂tr∂tr∂y+rz∂v∂tr∂tr∂z |  |
|  | r∙∇v=ar∙∇tr | 13.9 |
|  | Term 2 of Eqn 13.8 v∙∇r=vx∂∂x+vy∂∂y+vz∂∂zr |  |
|  | =vx∂r∂x+vy∂r∂y+vz∂r∂z |  |
|  | =vx∂r∂tr∂tr∂x+vy∂r∂tr∂tr∂x+vz∂r∂tr∂tr∂x |  |
|  | v∙∇r=vv∙∇tr | 13.10 |
|  | Cross product Term 3 of Eqn 13.8 ∇×v=∂vz∂y-∂vy∂zx+∂vx∂z-∂vz∂xy+∂vy∂x-∂vx∂yz |  |
|  | =∂vz∂tr∂tr∂y-∂vy∂tr∂tr∂zx+∂vx∂tr∂tr∂z-∂vz∂tr∂tr∂xy+∂vy∂tr∂tr∂x-∂vx∂tr∂tr∂yz |  |
|  | ∇×v=-a×∇tr | 13.11 |
|  | Cross product Term 4 of Eqn 13.8 ∇×r=∇×r-wtr |  |
|  | =∇×r-∇×wtr |  |
|  | Substituting ∇×r=0 ∇×r=-∇×w |  |
|  | =-∂wz∂y-∂wy∂zx+∂wx∂z-∂wz∂xy+∂wy∂x-∂wx∂yz |  |
|  | =-∂wz∂tr∂tr∂y-∂wy∂tr∂tr∂zx+∂wx∂tr∂tr∂z-∂wz∂tr∂tr∂xy+∂wy∂tr∂tr∂x-∂wx∂tr∂tr∂yz |  |
|  | ∇×r=v×∇tr | 13.12 |

## Inputs for implementation

The inputs for implementation are the path of the particle, frequency of oscillation and number of cycles. Path options will be closed curve or open curve. The path extend will be scaled to unit square.